# HyperCube Evolution Duet 2 vs Duet 3

While a Duet 3 won’t make your 3d printer print better such as direct quality improvements. It does add a potential range of options and functionality.

Although these options are not for everyone. You do get much more out of Duet by using an rpi.

Users can take advantage of many options when adding a RPi.

Duet3 is a professional board and maybe overkill for the home hobbyists but it's a nice way to set up firmware.

For example:

- You can mount networking shares to access shared files for all machines on render printer farms.

- It offloads the duet so it can focus on executing the gcode’s which means faster printing potential.

- You can directly update/upgrade Duet instead of fiddling around for the right files on Github.

- Using a pi you can use a much cheaper and better touch screen than PanelDue.

- Using rpi you use the new Duet Software Framework (DSF) which runs on rpi and opens up a lot of options.

- Easier for 3rd party developers since the single board connection is separate from the main board

| Duet 2 | Duet 3 |
| --- | --- |
| 12 Stepper Motors (with expansion board) | 24+ Stepper Motors |
|  | Single Board Computer Connection (RasberiPi) |
| 2.4 AMP Peak | 6.3 AMPS Peak |
|  | 5160 Drivers |
|  | CAN Bus |
| Better documentation | Plugins |
|  | More Expandability Trinamic 5160 stepper drivers Tool Boards |

Other than connectivity with the Duet 3 it doesn’t offer any practical functionality or printing improvement. It's the conditional gcode and the ability to use different pin assignments in RRF3 that makes it different, but it actually isn't that different but the terminology requires a more in depth explanation than RRF2.

# Duet3D YAT Commands

M115 Checks Firmware

M552 Checks Ethernet

M552 S0 Stops Ethernet

M552 S1 Starts Ethernet (displays current IP address)

M552 S1 P0.0.0.0 Sets Dynamic IP (P=desired ip address)

M552 S1 Pxxxxxx Sets Static IP

# Reverse Motor Direction

M569 command is for the motors in the config.g file.

Reprap Gcode M569:_Set_axis_direction_and_enable_values

- M569 P0 is for the X driver,

- M569 P1 is for the Y driver

- Change S1 in that line to S0 or vice versa to reverse the motor direction.

# To Invert Motors: On Config.g file

M569 P0 S0 ; Drive 0 goes forwards
M569 P1 S0 ; Drive 1 goes forwards.

M569 P2 S0 ; Drive 2 goes forwards
M569 P3 S1 ; Drive 3 goes forwards
M569 P4 S1 ; Drive 4 goes forwards

## Motor Drives are mapped as:

0 = X

1 = Y

2 = Z

3 = E0

4 = E1 etc

## “S” Parameter changes motor direction:

EX.

Change

M569 P3 S1

To

M569 P3 S0

Make sure your homex.g / homey.g / homez.g files home in the right direction:

Ex. X-Endstop positioned on the right side.

Change

; homex.g

; called to home the X axis

;

;G91               ; relative positioning

G1 H2 Z5 F6000    ; lift Z relative to current position

G1 H1 X-505 F1800 ; move quickly to X axis endstop and stop there (first pass)

G1 H2 X5 F6000    ; go back a few mm

G1 H1 X-505 F360  ; move slowly to X axis endstop once more (second pass)

G1 H2 Z-5 F6000   ; lower Z again

G90               ; absolute positioning

To

; G91               ; relative positioning

G1 H2 Z5 F6000    ; lift Z relative to current position

G1 H1 X505 F1800 ; move quickly to X axis endstop and stop there (first pass)

G1 H2 X-5 F6000    ; go back a few mm

G1 H1 X505 F360  ; move slowly to X axis endstop once more (second pass)

G1 H2 Z-5 F6000   ; lower Z again

G90               ; absolute positioning

Also See:

WorkHorse Files

config.g

Homeall.g

homex.g

Homey.g

Homez.g

Duet 3 Mainboard Setup

Duet3D YAT Commands

Reverse Motor Direction

## Duet Wifi

Duet Wifi Setup

Duet 2 vs Duet 3

## See More

3D Distributed

WorkHorse 3D Printer

DIY Large ScaleHyperCube Evolution  3D Printer

Large Scale 3D Printers

## Workhorse 3D Printer-Firmware

| RepRap Firmware |  |
| --- | --- |
| config.g |  |
| Homeall.g |  |
| homex.g |  |
| Homey.g |  |
| Homez.g |  |