# Homeall.g

; homeall.g

; called to home all axes

;

; generated by RepRapFirmware Configuration Tool v3.1.4 on Mon Oct 26 2020 01:06:05 GMT-0500 (Central Daylight Time)

G91                     ; relative positioning

G1 H2 Z5 V5 F6000          ; lift Z relative to current position

G1 H1 X505 U505 Y505 F1800 ; move quickly to X and Y axis endstops and stop there (first pass)

G1 H2 X-5 U-5 Y-5 F6000       ; go back a few mm

G1 H1 X505 U505 Y505 F360  ; move slowly to X and Y axis endstops once more (second pass)

G1 H1 Z-405 V-405 F360        ; move Z down stopping at the endstop

G90                     ; absolute positioning

G92 Z0 V0                ; set Z position to axis minimum (you may want to adjust this)

; Uncomment the following lines to lift Z after probing

;G91                    ; relative positioning

;G1 Z5 F100             ; lift Z relative to current position

;G90                    ; absolute positioning

Also See:

3D Distributed

WorkHorse 3D Printer

DIY Large Scale 3D Printer

Large Scale 3D Printers

WorkHorse Files

config.g

Homeall.g

homex.g

Homey.g

Homez.g

Duet 3 Mainboard Setup

Duet3D YAT Commands

Reverse Motor Direction

Duet Wifi

Duet Wifi Setup