# Getting Connected To Duet Wifi

Use this for your Workhorse Printer Setup but with Duet 3 Setup

- Connect the Duet to your PC with the USB cable provided. Do not connect other power supplies at this stage.

- Windows users, download the USB drivers for Duet here and extract the files to a suitable location.

- Install Drivers.

- Download YAT Terminal Interface.

- Setup YAT to use <LF> as the end of line character. Go to "Terminal" Menu -> "Settings" -> "Text Settings" -> "EOL sequence:" set to <LF>

- Windows Users: Open YAT, and configure the correct COM port.

- send command M115

- The WiFi module has a blue LED on it, just next to the Micro SD card holder. This should flash once when power is connected to the board, but will only be lit permanently when connected to a WiFi network. Duets with an external antenna use a different WiFi module with no LED; if you have one of these, don't forget to plug in the external antenna!

- Send M552 to check the status. It should say "WiFi module is disabled". Now put it into idle mode by sending command M552 S0. Send M552 again to check it's idle, then proceed to the next step.

| Duet Wifi vs Duet 3 |  |
| --- | --- |
| Duet Wifi Setup |  |
| Stepper Motor Holding Torque |  |
| Unipolar vs Bipolar Stepper Motors |  |
| 0.9 vs 1.8 degrees Stepper Motors |  |
| Duet Wifi |  |
| Stepper Motors |  |
| Duet Wifi Setup |  |
| TripleZ Setup |  |
| Triple Independent Motors Z axis |  |
| TripleZ Setup |  |
| Duet3D YAT Commands |  |
| Getting Connected To Duet Wifi |  |
| Duet 2 vs Duet 3 |  |
| Electronics |  |
| Duet3D |  |
| Duet 3 |  |
| Duet 3 Reverse Motor Direction |  |
| Duet 3 Mainboard Config.g Reprap Firmware File |  |
| CoreXY |  |
| 3D Distributed |  |
| Large Scale 3D Printer |  |
| Duet Wifi Reprap Firmware |  |
| Set Macro |  |
| M569: Set motor driver direction |  |
| Duet won't respondd |  |
| SolidCore CoreXY |  |
| Workhorse 3D Printer |  |
| RepRap Firmware |  |
| config.g |  |
| Homeall.g |  |
| homex.g |  |
| Homey.g |  |
| Homez.g |  |

## Related Content

3D Distributed

Duet Wifi Setup

Stepper Motor Holding Torque

Unipolar vs Bipolar Stepper Motors

0.9 vs 1.8 degrees Stepper Motors

3D Distributed

DIY 3D Printers

3D Printer Parts

3D Printer Kits

Large Scale 3D Printers

CoreXY

Midwest RepRap Festival

12601 Bowles Rd. Conroe, TX 77304

SolidCore CoreXY Articles

Belts Crossed

Belt Tensioning

Voron vs D-Bot

Duet3D YAT Commands

HyperCube Evolution

CoreXY Explained

Is CoreXY Better?

SolidCore Parts

Crossed Belts vs Offsetting Motors

Pulley Distance

TripleZ

TripleZ Setup

SC Motor Mount Machining Operations

SolidCore

SolidCore : Independent Z-Axis and Gantry Desinn

CoreXY Cool

SolidCore CoreXY Articles

Bed Plate Options

What You Need to Know Before Buying a 3D Printer

SolidCore Bed Mount

SolidCore Kit Options

How To Reduce Ringing

SolidCore Questions

Stepper Motors

Z-Axis Bed Mount

Heat Bed Build Plate Magnets

ToolChanger

3D Distributed

SolidCore CoreXY

SolidCore Z-Axis

CoreXY Motor Mounts

Y-Carriage

CoreXY Kinematics

Houston