# What to do if your Duet won't respond

#### Check the voltage indication LEDs

- If you have a PanelDue, disconnect it from the Duet to reduce the load on the 5V supply. Then connect your Duet to a PC using the USB cable. Do not apply VIN power to the Duet at this point.

- Check the LEDs on the Duet. There should be a red LED illuminated between the USB connector and the Reset button. The red 5V LED at the edge of the board should also be lit, so should the green 3.3V LED. (Note: the Duet 06 doesn't have the 5V or 3.3V LED)

- If the red LEDs are lit but not the green 3.3V LED then most likely there is a short between the 3.3V power rail and ground. Disconnect the Duet from USB, then remove all endstop and Z probe connectors from the Duet. If you have a Duex 2 or Duex 5 connected as well, disconnect the ribbon cable joining them too. Then reconnect the USB cable and check the LEDs again.

- The DIAG LED between the USB connector and the SD card socket may or may not be lit, but can help diagnosis. (Note: the Duet 06/085 doesn't have the DIAG LED.)

  - In normal operation, the DIAG LED will turn on as power is applied, then turn off as soon as the firmware starts loading, usually less than 0.5 second.

  - In normal operation, it will turn back on if the probe type is set to M558 P1 in config.g. Removing the SD card and resetting should switch it off. Apart from this case, the DIAG LED should normally be off.

  - Errors in config.g may cause the Duet to get stuck in a boot loop, which will cause the DIAG LED to be on permanently. Remove SD card and reset.

  - If the reset button is stuck in (rare), or has been mounted so it is pushed in by an enclosure, the Duet will get stuck in a boot loop, which will cause the DIAG LED to be on permanently. Check there is clearance around the reset button, or use some tweezers to gently pull the button out.

  - If the firmware has been erased the DIAG LED will be on permanently.

- If no LEDs are lit, then most likely your PC is not supplying power through the USB cable. Try a different cable and a different USB port. If necessary, use a powered USB hub.

- Remove the SD card, as a corrupt or incorrectly configured config.g file can cause the firmware to continually reboot. There have also been occasions where the SD card causes a short circuit.

| RepRap Firmware |  |
| --- | --- |
| config.g |  |
| Homeall.g |  |
| homex.g |  |
| Homey.g |  |
| Homez.g |  |

Related Content:

WorkHorse Files

config.g

Homeall.g

homex.g

Homey.g

Homez.g

Duet 3 Mainboard Setup

Duet3D YAT Commands

Reverse Motor Direction

Duet Wifi

Duet Wifi Setup

Duet 2 vs Duet 3

## See More

3D Distributed

WorkHorse 3D Printer

DIY Large Scale 3D Printer

Large Scale 3D Printers

Crossed Belts vs Offsetting Motors

CoreXY Explained

SolidCore X-Carriage

CoreXY 3D Printer

Belt Tensioning

Voron vs D-Bot

## Related Content

3D Distributed

Duet Wifi Setup

What To Do If Duet Won’t Respond

Set motor driver direction

Set Macro

Duet3D

| Duet Wifi vs Duet 3 |  |
| --- | --- |
| Duet Wifi Setup |  |
| Stepper Motor Holding Torque |  |
| Unipolar vs Bipolar Stepper Motors |  |
| 0.9 vs 1.8 degrees Stepper Motors |  |
| Duet Wifi |  |
| Stepper Motors |  |
| Duet Wifi Setup |  |
| TripleZ Setup |  |
| Triple Independent Motors Z axis |  |
| TripleZ Setup |  |
| Duet3D YAT Commands |  |
| Getting Connected To Duet Wifi |  |
| Duet 2 vs Duet 3 |  |
| Electronics |  |
| Duet3D |  |
| Duet 3 |  |
| Duet 3 Reverse Motor Direction |  |
| Duet 3 Mainboard Config.g Reprap Firmware File |  |
| CoreXY |  |
| 3D Distributed |  |
| Large Scale 3D Printer |  |
| Duet Wifi Reprap Firmware |  |
| Set Macro |  |
| M569: Set motor driver direction |  |
| Duet won't respondd |  |
| SolidCore CoreXY |  |
| Workhorse 3D Printer |  |

| 3D Distributed |  |
| --- | --- |
| DIY 3D Printers |  |
| 3D Printer Parts |  |
| 3D Printer Kits |  |
| Large Scale 3D Printers |  |
| CoreXY |  |
| Midwest RepRap Festival |  |
| 12601 Bowles Rd. Conroe, TX 77304 |  |