# How To Calculate Stepper Motor RPM?

## Formula for calculating stepping motor speed.

Below is the RPM formula for stepping motor drives that use pulse signals to create motion.

RPM = a/360 * fz * 60

RPM = Revolutions per minute.

“a” = step angle

“fz” = pulse frequency in hertz

Example 1:  Drive step resolution is set for 1000 steps per revolution.

Find step angle: 360 (degrees of rotation) divided by 1000 (steps per revolution) = .36 (degrees of rotation per step)

With input frequency of 1000hz, .36 / 360 * 1000 * 60 = 60 rpm

Example 2: Drive step resolution is set for 500 steps per revolution.

Find step angle: 360 (degrees of rotation) divided by 500 (steps per revolution) = .72 (degrees of rotation per step)

With input frequency of 1000hz, .72 / 360 * 1000 * 60 = 120 rpm.

To find required frequency to meet desired speed we must find Fz:

RPM / ( (a/360) * 60) = Fz

Step 1. Divide step angle by 360:  .36/360 = .001,  then multiply by 60 (seconds) .001 * 60 = .06

Step 2. Divide desired speed by .06: Desired speed of 500 rpm’s.   Therefore ; 500 / .06 = 8333.33   ( 8333.33 is the frequency in hertz required to reach 500 rpm’s for drive set at 1000 steps per revolution)

# Related Content

## WorkHorse 3D Printer

## Duet3D

## Electronics

## Stepper Motor Holding Torque

## Electronics

## Duet 3

## Duet 3 Wiring Guide

## Duet Wifi

## RepRap Firmware

# Docs

## Stepper Motors

## How To Calculate Stepper Motor RPM?

## Stepper Motor Holding Torque

## Unipolar vs Bipolar Stepper Motors

## EndStops

## 0.9 vs 1.8 degrees Stepper Motors

## WorkHorse-3D Printer Wiring

## Set Macro Explained

## Dual X-Axis / Dual Z