# Unipolar vs Bipolar Stepper Motors

## Bipolar Stepper Motor

- 4 wires

- 6 wires

- 8 wires

## Unipolar Stepper Motors

- 5 wires

### You cannot use motors with 5 wires, because those are intended to be driven in unipolar mode only. (Some unipolar motors can be made into bipolar motors by cutting a trace on a circuit board.)

## 4 Wires

Inside the stepper motor are two coils, each coil having a wire connected to each end. The wire and coil pairs are called a phase. The 4 wires map to the 4 output pins of each stepper driver on the Duet.

## 6 Wires

With 6-wire stepper motors, there are still 2 coils, but each coil has a centre tap, effectively cutting the coil in half if needed. This creates an extra wire for each coil. You can choose to run them in half-coil, by leaving two end-wires unconnected, or full coil mode by leaving the centre wires unconnected. See the motor specification to check that your Duet can supply enough current for how you want to wire them.

## 8 Wires

The 8-wire stepper has 4 coils, so with two wires per coil, this makes 8 wires.

- An 8-wire stepper in half-coil with only 2 coils connected or full-coil mode

- Full-coil mode you can choose to wire the coils in series or in parallel.

- 4 wires are needed to connect to the Duet.

## Rated current

This is the maximum current you may pass through both windings at the same time. The maximum current through one winding (which is what really matters when using microstepping) is rarely quoted and will be a little higher. However, even with one winding driven at the quoted rated current, the motor will get very hot. So the usual practice is to set the motor current to no more than about 85% of the rated current. Therefore, to get maximum torque out of your motors without overheating them, you should choose motors with a current rating no more than 25% higher than the recommended maximum stepper driver current. This gives:

- Duet 0.6 and Duet 0.8.5 (recommended maximum motor current 1.5A peak) => Stepper motor rated current <= 1.9A

- Duet 2 WiFi and Duet 2 Ethernet (maximum motor current 2.5A peak) => Stepper motor rated current <= 3.0A

- Duet 2 Maestro (maximum motor current 1.6A peak with good fan cooling) => Stepper motor rated current <= 1.7A. Hower, if you use motors with lower rated current (e.g. 1.0 to 1.2A) and 24V power, then the drivers will run cooler.

- Duet 3 Mainboard 6HC and Expansion board 3HC (recommended maximum motor current 6.3A peak/4.45A RMS) => Stepper motor rated current <= 6A

- Duet 3 Tooboard (recommended maximum motor current 1.4A peak) => Stepper motor rated current <= 1.75A

# Related Content

## WorkHorse 3D Printer

## Duet3D

## Electronics

## Stepper Motor Holding Torque

## Electronics

## Duet 3

## Duet 3 Wiring Guide

## Duet Wifi

## RepRap Firmware

# Docs

## Stepper Motors

## How To Calculate Stepper Motor RPM?

## Stepper Motor Holding Torque

## Unipolar vs Bipolar Stepper Motors

## EndStops

## 0.9 vs 1.8 degrees Stepper Motors

## WorkHorse-3D Printer Wiring

## Set Macro Explained

## Dual X-Axis / Dual Z