# Workhorse Printer

WorkHorse Printer

The Workhorse Printer is a large format, open source 3d printer released by 3D Distributed in 2018. The printer is a modular machine platform that uses lead screw driven motion with a fixed bed / moving gantry.

- Fixed Bed / Moving Gantry

- X and Y-Axis Lead Screw

Most 3d printers use belt driven motion in the x and y axis to increase print speed but loses quality when more weight is applied to the gantry or carriage. The lead screw used on the Workhorse has a high helix pitch with multiple threads to allow increased print speeds while maintaining the quality provided from lead screw and ball screw motion.The robust and rigid system is particularly useful in applications that add more weight to the gantry or carriage. Materials such as clay, chocolate, plastic pellet extrusion or other foods can increase weight that is moved around which results in decreased quality or print speeds. Workhorse Printer’s are available to purchase in a Machined Component Kit, Hardware Kit or Complete Kit.

Workhorse 3D Printer MRRF 2019

- OPEN SOURCE

- LARGE BUILD VOLUME

- ADVANCED LEAD SCREW MOTION

- MODULAR MACHINE DESIGN

- CUSTOM MODS

- CUSTOM BUILDS

- ENCLOSURE

- HEATED BED

## Carriage

Large 3D Printer Kit

The Workhorse Printer’s C-Carriage design is a robust solution for lead screw driven motion that uses linear rails and 20×40 or 20×20 aluminum extrusions.

WorkHorse Carriage

The Workhorse 3d printer is available with custom mods and upgrades. We’ve several custom designed machines for many of our customers. If there is a specific size or requirement you just give us a shout.

## New Heated Bed

## WorkHorse Printer Updates

## Download Files

## WorkHorse Printer CAD

### Thingiverse Files

## 3D Distributed Parts

WorkHorse 3D Printer Parts

WorkHorse 3D Printer Gantry Plates

| Machined Hardware | Price $ | Source |
| --- | --- | --- |
| 4″ plate kit 2pcs w/bearings |  | 3DD |
| X- Axis C- Carriages 2pcs |  | 3DD |
| Y-Axis C- Carriage 1pc |  | 3DD |
| Y-Axis Motor Mount 1pc |  | 3DD |
| Y-Axis Idler Mount 1pc |  | 3DD |
| Hotend Mount |  | 3DD |
| Z-Axis Motor Mounts 2pcs |  | 3DD |
| Aluminum Bed |  |  |

WorkHorse BOM

## Duet3D Firmware Files

Download

## Electronics

We now recommend the Duet 3 Mainboard 6HC electronics by Duet3D.

## Build Instructions

Here

Share this:

| 3D Distributed |  |  |  |  |
| --- | --- | --- | --- | --- |
| WorkHorse 3D Printer |  |  |  |  |
| Electronics | Price Each | Part Number | Pcs. |  |
| Nema 17 Stepper Motor Z-Axis | 17.95 | MO-N17-48-22B-3X | 2pcs |  |
| NEMA 23 269 oz.in X-Axis | 17.95 | MO-N23-76-21B | 2pcs |  |
| NEMA 23 184 oz.in Y -Axis | 17.95 | KL23H251-28-4A | 1pcs |  |
| Duet 3 Mainboard | Duet 3D |  | 1pcs |  |
| ARCOL / Ohmite HS50 300R F |  |  | 30 |  |
| Omron Automation and Safety G3NA-220B-UTU AC100-120 |  |  | 1 |  |
| Lead Screw | Price $ Each | Part Number | Description |  |
| Igus 761mm Long (X Axis) | $ 93.23 | DST-LS-10x50-R | DST-LS-10X50-R-ES |  |
| Igus 485mm Long (Y Axis) | $ 74.80 | DST-LS-10x50-R | DST-LS-10X50-R-ES |  |
| Igus Lead Screw Nuts | $ 35.78 | DST-JFRM-2525DS1 0X50 | Ds10x50 |  |
| 12mm T12x2 Trapezoidal ACME Lead Screw w/ Brass Nut |  |  |  |  |
| Total | $ 368.60 |  |  |  |
| Linear Rails | Price $ Each | Discription | Source |  |
| 450mm or 400mm Long w Y axis (1pc) | $ 61.95 | MGN12H 20mm x 20mm Hole Pattern | ZYLtech |  |
| 700mm Long X Axis (2pc) | $ 29.95 | MGN12H 20mm x 20mm Hole Pattern | ZYLtech |  |
| 500mm Long Z Axis (4pc) | $ 44.95 | MGN12H 20mm x 20mm Hole Pattern | ZYLtech |  |
| Total | $ 301.65 |  |  |  |
| Total w/shipping + Tax | $361.17 |  |  |  |
| Machined Hardware | Price $ | Source |  |  |
| Gantry Plates 2pcs w/bearings | $ 65.00 | 3D Distributed |  |  |
| X- Axis Carriages 2pcs | $ 44.80 | 3D Distributed |  |  |
| Y-Axis Carriage 1pc | $ 19.95 | 3D Distributed |  |  |
| Y-Axis Motor Mount 1pc | $ 15.50 | 3D Distributed |  |  |
| Y-Axis Idler Mount 1pc | $ 17.50 | 3D Distributed |  |  |
| Hotend Mount | $ 16.00 | 3D Distributed |  |  |
| Z-Axis Motor Mounts 2pcs | $ 37.50 | 3D Distributed |  |  |
| Aluminum Bed | $ 100.00 | 3D Distributed |  |  |
| Total | $ 316.25 |  |  |  |
| Frame Extrusions | Length | Units | Description | Part # |
| HFS5-2020 | 930mm Long | 4pcs | Horizontals | HFS5-2020-930 |
| HFS5-2020 | 510mm Long | 6pcs | Verticals | HFS5-2020-510 |
| HFS5-2020 | 570mm Long | 10pcs | Widths | HFS5-2020-570 |
| HFS5-2020 | 808mm Long | 3pcs | Bed Support | HFS5-2020-808 |
| Gantry Extrusions | Length | Units | Description | Part# |
| HFS5-2040 | 770.5 Long | 2pcs | Gantry Horizontals | HFS5-2040-770 |
| HFS5-2040 | 456Long | 1pcs |  |  |
| HFS5-2040 | 510 Long | 4pcs. | Z axis Linear Rail Verticals | HFS5-2040-510 |
| Frame Components | Type | Units | Description | Part# |
| Frame Brackets | 20mm Wide | 90pcs | Reversal Tabbed Brackets | HBLFSNF5 |
| Frame Brackets | M4 | 75pcs | M4 | HNKK5-4 |
| Frame Brackets | M5 | 180pcs | M5 | HNKK5-5 |
| Frame Brackets | M3 | 75pcs | M3 | HNKK5-3 |
| Frame Price | Price $ | Estimated Shipping |  |  |
| Profile Extrusions | $ 122.29 |  | extrusions and backets w tax and shipping |  |
| M5 x 10 Cap Screws (200pcs) | $ 9.99 |  | 225$ |  |
| brackets | $ 40.32 |  |  |  |
| T-nuts | $ 19.32 |  |  |  |
| T-nuts M3 (100pcs linear rail mounting) HNKK5-3 | $ 17.71 | 11.99 |  |  |
| Total | $ 191.92 |  |  |  |

## Related Content

3D Distributed

WorkHorse 3D Printer

SolidCore CoreXY 3D Printer

Duet3D

WorkHorse 3D Printer BOM

WorkHorse 3D Printer

WorkHorse CAD

3D Distributed

DIY 3D Printers

3D Printer Parts

3D Printer Kits

Large Scale 3D Printers

CoreXY

Midwest RepRap Festival

All Metal 3D Printer Parts

12601 Bowles Rd. Conroe, TX 77304