# config.g

; Configuration file for Duet 3 (firmware version 3)

; executed by the firmware on start-up

;

; generated by RepRapFirmware Configuration Tool v3.1.4 on Mon Nov 02 2020 15:14:30 GMT-0600 (Central Standard Time)

; General preferences

G90                                                ; send absolute coordinates...

M83                                                ; ...but relative extruder moves

M550 P"WorkHorse XL"                                     ; set printer name

M552 P192.168.0.21 S1                                ;enable network and set ip

; Drives

M569 P0 S0                                       ; physical drive 0.0 goes forwards

M569 P1 S0                                       ; physical drive 0.0 goes forwards

M569 P2 S0                                      ; physical drive 0.2 goes forwards

M569 P3 S1                                      ; physical drive 0.3 goes forwards

M569 P4 S1                                     ; physical drive 0.0 goes forwards

M569 P5 S1                                       ; physical drive 0.5 goes forwards

M584 X0:1 U1 Y2 Z3:4 V4 E5                           ; set drive mapping

M350 X16 U16 Y16 Z16 V16 E16 I1                            ; configure microstepping with interpolation

M92 X64.00 U64.00 Y64.00 Z1259 V1259 E420.00                  ; set steps per mm

M566 X1200 U1200 Y1200 Z12.00 V12.00 E120.00                ; set maximum instantaneous speed changes (mm/min)

M203 X7200 U7200 Y7200 Z200 V200 E1200.00            ; set maximum speeds (mm/min)

M201 X500.00 U500.0 Y500.00 Z20.00 V20.00 E250.00                ; set accelerations (mm/s^2)

M906 X2800 U2800 Y2800 Z2800 V2800 E800 I30                       ; set motor currents (mA) and motor idle factor in per cent

M84 S30                                            ; Set idle timeout

; Axis Limits

M208 X0 U0 Y0 Z0 V0 S1                                   ; set axis minima

M208 X500 U500 Y500 Z500 V500 S0                             ; set axis maxima

; Endstops

M574 X2 S1 P"io1.in"                               ; configure active-high endstop for low end on X via pin io1.in

M574 U2 S1 P"io2.in"                               ; configure active-high endstop for low end on X2 via pin io1.in

M574 Y2 S1 P"io3.in"                               ; configure active-high endstop for low end on Y via pin io3.in

M574 Z2 S1 P"io4.in"                               ; configure active-high endstop for low end on Z via pin io4.in

M574 V2 S1 P"io5.in"                               ; configure active-high endstop for low end on z2 via pin io1.in

; Z-Probe

M558 P0 H5 F120 T6000                              ; disable Z probe but set dive height, probe speed and travel speed

M557 X15:215 Y15:195 S20                           ; define mesh grid

; Heaters

M308 S0 P"temp3" Y"thermistor" T100000 B4138       ; configure sensor 0 as thermistor on pin temp3

M950 H0 C"out2" T0                                 ; create bed heater output on out2 and map it to sensor 0

M307 H0 B1 S1.00                                   ; enable bang-bang mode for the bed heater and set PWM limit

M140 H0                                            ; map heated bed to heater 0

M143 H0 S120                                       ; set temperature limit for heater 0 to 120C

M308 S1 P"temp0" Y"thermistor" T100000 B4138       ; configure sensor 1 as thermistor on pin temp0

M950 H1 C"out0" T1                                 ; create nozzle heater output on out0 and map it to sensor 1

M307 H1 B0 S1.00                                   ; disable bang-bang mode for heater  and set PWM limit

; Fans

M950 F0 C"out8" Q500                               ; create fan 0 on pin out8 and set its frequency

M106 P0 S0 H-1                                     ; set fan 0 value. Thermostatic control is turned off

M950 F1 C"out9" Q500                               ; create fan 1 on pin out9 and set its frequency

M106 P1 S1 H1 T45                                  ; set fan 1 value. Thermostatic control is turned on

; Tools

M563 P0 D0 H1 F0                                   ; define tool 0

G10 P0 X0 Y0 Z0                                    ; set tool 0 axis offsets

G10 P0 R0 S0                                       ; set initial tool 0 active and standby temperatures to 0C

; Custom settings are not defined

; Miscellaneous

M911 S10 R11 P"M913 X0 Y0 G91 M83 G1 Z3 E-5 F1000" ; set voltage thresholds and actions to run on power loss

Also See:

3D Distributed

WorkHorse 3D Printer

DIY Large Scale 3D Printer

Large Scale 3D Printers

WorkHorse Files

config.g

Homeall.g

homex.g

Homey.g

Homez.g

Duet 3 Mainboard Setup

Duet3D YAT Commands

Reverse Motor Direction

Duet Wifi

Duet Wifi Setup